HAPS FSO PROJECT

From OOK Transmission to Optical Jamming, Receiver Blinding and the ML Transition


Coverage: Baseline → EXP 11
Simulation Platform: MATLAB
System: HAPS-to-Ground Free-Space Optical Communication

Prepared from the actual project source archive and recorded experiment outputs.

# Document Purpose and Reading Guide

This document is the master technical record of the HAPS FSO project through Experiment 11. It explains the system from first principles, maps every major MATLAB module to the underlying physics, documents the experiments and reconstructs the central results and patterns observed during development.

The primary source is the uploaded MATLAB project archive. Numerical plots for EXP 09–11 are reconstructed from recorded experiment outputs. Representative baseline turbulence values come from the baseline consolidated documentation contained in the project archive.

## How to read this document

| Reader | Recommended path |
| --- | --- |
| Beginner | Sections 1 → 7 → 10 → 14 → 20 |
| MATLAB reviewer | Sections 3 → 13 and Appendix A |
| Research reviewer | Sections 14 → 24 |
| Future ML developer | Sections 20 → 25 |

## Project status at the end of this document

The legitimate HAPS FSO baseline is modularized and validated.

Constant and random optical jammer models are implemented separately.

Constant-jammer power sweeps reveal a non-monotonic BER curve and a blinding transition.

Random intermittent jamming is validated and swept by activity probability.

EXP 11 compares constant and random jammers under the same average power budget.

The next phase is ML-based attack detection, classification and mitigation.

# Table of Contents

1. Project Overview and Research Story

2. High-Level End-to-End Architecture

3. Project Repository and Software Architecture

4. Default System Parameters

5. Binary Information and OOK Transmission

6. Geometrical Propagation

7. Atmospheric Attenuation

8. Pointing Error

9. Gamma-Gamma Atmospheric Turbulence

10. Combined Legitimate FSO Channel

11. Receiver

12. runBaseline Simulation Engine

13. Testing and Baseline Validation

14. Baseline Experiments EXP_BASE_01 to EXP_BASE_04

15. Threat Model: Optical Jamming

16. Constant Jammer and EXP 05–06

17. Random Jammer and EXP 07–08

18. Constant Blinding Attack: EXP 09

19. Random Blinding Attack: EXP 10

20. Fair Comparison: EXP 11

21. Cross-Experiment Patterns

22. Reproducibility and Debugging Lessons

23. Model Assumptions and Limitations

24. Research Conclusions

25. Bridge to Machine Learning

Appendix A. File-by-File Code Map

Appendix B. Key Equations

Appendix C. Key Experimental Results

# 1. Project Overview and Research Story

The project studies a HAPS-to-ground free-space optical (FSO) communication link using intensity modulation/direct detection. A binary source is converted into an on-off optical waveform, propagated through deterministic and random atmospheric effects, converted into receiver current, and detected with a fixed threshold. After the legitimate baseline is established, an adversarial optical source is added to study jamming and receiver blinding.

## Research progression

Random bits
   ↓
OOK optical transmission
   ↓
Gaussian-beam geometrical coupling
   ↓
Atmospheric attenuation
   ↓
Pointing loss + Gamma-Gamma turbulence
   ↓
Received legitimate optical signal
   ↓
Photodetection + receiver noise + optional saturation
   ↓
Threshold decision → BER
   ↓
Constant optical jammer
   ↓
Random intermittent jammer
   ↓
Receiver blinding studies
   ↓
Fair constant-vs-random comparison
   ↓
ML detection / classification / mitigation

## Core questions

How do geometry, attenuation, pointing error and turbulence affect an OOK HAPS FSO link?

How does constant optical jamming alter BER and error type?

Why can small jammer power temporarily improve BER in this fixed-threshold model?

How does intermittent jamming differ from continuous jamming?

How does receiver saturation create a blinding regime?

Under the same average power budget, which temporal attack structure is more damaging?

Which receiver signatures should feed the next ML phase?

# 2. High-Level End-to-End Architecture

[Random Bits]
      ↓
[OOK Transmitter]
      ↓
[Gaussian Beam Geometry] ─ H_geo
      ↓
[Atmospheric Attenuation] ─ H_atm
      ↓
[Pointing Error] ───────── H_point(k)
      ↓
[Gamma-Gamma Turbulence] ─ H_turb(k)
      ↓
[Legitimate Received Optical Power]
      ↓
      + P_jammer(t) when attacked
      ↓
[Photodetector]
      ↓
[Receiver Noise]
      ↓
[Optional Saturation]
      ↓
[Mid-bit Sampling]
      ↓
[Threshold Detector]
      ↓
[Detected Bits + BER]

P_rx,legit = P_tx · H_geo · H_atm · H_point · H_turb

P_rx,total = P_rx,legit + P_jammer

The legitimate channel is multiplicative. The adversarial jammer is additive at the optical input before photodetection.

# 3. Project Repository and Software Architecture

HAPS_FSO_PROJECT/
├── baseline_model/
│   ├── defaultParameters.m
│   ├── main_baseline.m
│   ├── runBaseline.m
│   └── MainModels/
│       ├── generateOOK.m
│       ├── geometryModel.m
│       ├── atmosphericAttenuation.m
│       ├── pointingErrorModel.m
│       ├── gammaGammaTurbulence.m
│       └── receiverModel.m
├── baseline with jammer/
│   ├── jammerModel.m
│   └── randomJammerModel.m
├── experiments/
│   ├── EXP_BASE_01_distanceSweep.m
│   ├── EXP_BASE_02_turbulenceSweep.m
│   ├── EXP_BASE_03_pointingSweep.m
│   ├── EXP_BASE_04_channelRealization.m
│   ├── EXP_05_constantJammer.m
│   ├── EXP_06_jammerPowerSweep.m
│   ├── EXP_07_randomJammerValidation.m
│   ├── EXP_08_randomJammerSweep.m
│   ├── EXP_09_constantJammerBlindingAttack.m
│   ├── EXP_10_randomJammerBlindingAttack.m
│   └── EXP_11_fairJammerComparison.m
└── tests/
    ├── test01_perfectChannel.m
    └── test02_receiverNoise.m

| Layer | Responsibility |
| --- | --- |
| Parameters | Nominal simulation settings. |
| Transmitter | Bits, OOK symbols and waveform. |
| Propagation | Geometry and atmospheric attenuation. |
| Random channel | Pointing and turbulence. |
| Receiver | Photodetection, noise, saturation, sampling and decision. |
| System engine | End-to-end legitimate simulation and diagnostics. |
| Attack layer | Constant and random jammer generation. |
| Experiments | Sweeps, comparisons and plots. |
| Tests | Sanity and sensitivity validation. |

# 4. Default System Parameters

| Parameter | Symbol / meaning | Default |
| --- | --- | --- |
| Number of bits | Nbits | 1000 |
| Bit rate | Rb | 1×10^6 bit/s |
| Transmit power | Pt | 1 W |
| Samples/bit | samplesPerBit | 20 |
| Wavelength | λ | 1550 nm |
| Divergence half-angle | θ | 0.01 mrad |
| Receiver diameter | D_Rx | 0.2 m |
| Responsivity | R | 0.8 A/W |
| Distance | L | 20 km |
| Atmospheric attenuation | α | 0.1 dB/km |
| Pointing std | σ_point | 0.02 m |
| Turbulence | Cn² | 1×10^-16 |
| Noise std | σ_noise | 0.005 A |
| Saturation default | enableSaturation | false |
| Random seed | rngSeed | 1 |

T_b = 1/R_b = 1 μs

F_s = R_b · samplesPerBit = 20 MHz

The 20 samples per bit provide numerical waveform resolution. In the present implementation, pointing and turbulence are generated once per bit and repeated over that bit's samples.

# 5. Binary Information and OOK Transmission

## 5.1 Random bit source

bits = randi([0 1], 1, Nbits);

b_k ∈ {0,1}

## 5.2 On-Off Keying

ookSymbols = Pt * bits;

P_tx(k) = 0 for b_k=0, and P_t for b_k=1

OOK maps information directly to optical intensity: zero means no legitimate optical contribution in this simplified model; one means Pt.

## 5.3 Waveform expansion

txSignal = repelem(ookSymbols, samplesPerBit);

Each bit-level symbol is repeated across 20 samples so the waveform can be processed by the channel and receiver.

## 5.4 Time axis

Fs = Rb * samplesPerBit;
t = (0:length(txSignal)-1) / Fs;

# 6. Geometrical Propagation

geometryModel.m models Gaussian-beam spreading and the fraction of beam power captured by a finite circular receiver.

w_0 = λ/(πθ)

z_R = πw_0²/λ

w_Rx = w_0√(1+(L/z_R)²)

H_geo = 1−exp(−2R_Rx²/w_Rx²)

As propagation distance increases, the beam spreads. A finite receiver aperture therefore captures a changing fraction of the Gaussian optical power.

The baseline consolidated documentation recorded a representative 20 km operating point of H_geo ≈ 0.375821 and w_Rx ≈ 0.2060 m for the project parameterization.

# 7. Atmospheric Attenuation

L_km = L / 1e3;
H_atm = 10^(-alpha * L_km / 10);

H_atm = 10^(−αL_km/10)

This converts the dB/km attenuation coefficient into a linear optical power gain. The present model is path-averaged and deterministic.

H_total = H_geo · H_atm

At α=0.1 dB/km and L=20 km, the consolidated baseline documentation records H_atm ≈ 0.630957.

# 8. Pointing Error

x_error = sigmaPoint * randn(1,Nbits);
y_error = sigmaPoint * randn(1,Nbits);
r_error = sqrt(x_error.^2 + y_error.^2);
H_point = exp(-2*r_error.^2/wRx^2);

x_e,y_e ~ N(0,σ_point²)

r_e = √(x_e²+y_e²)

H_point(k)=exp(−2r_e(k)²/w_Rx²)

One random pointing loss is generated per bit and repeated across the samples of that bit. Severe misalignment reduces legitimate received power and increases the chance of 1→0 errors.

# 9. Gamma-Gamma Atmospheric Turbulence

gammaGammaTurbulence.m represents scintillation as a random multiplicative gain.

k = 2π/λ

σ_R² = 1.23 · Cn² · k^(7/6) · L^(11/6)

H_turb = X·Y

The implementation derives Gamma-Gamma parameters α_GG and β_GG from the Rytov variance, generates normalized Gamma variables X and Y, and multiplies them.

When Cn²=0, the source explicitly returns H_turb=1, giving a clean no-turbulence validation condition.

Representative baseline turbulence sweep trend documented in the project's consolidated baseline report.

The representative documented values rise from BER ≈0.0021 at Cn²=1×10^-17 to BER ≈0.2147 at Cn²=1×10^-15. Exact values depend on random realizations.

# 10. Combined Legitimate FSO Channel

P_rx(t)=P_tx(t)·H_geo·H_atm·H_point(t)·H_turb(t)

P_rx(t)=P_tx(t)·H_total·H_point(t)·H_turb(t)

rxOpticalSignal =
    H_total .* H_point_signal .* H_turb_signal .* txSignal;

For bit 0, the legitimate optical contribution is ideally zero. For bit 1, received power fluctuates because pointing and turbulence vary.

A critical design choice is that the receiver threshold uses H_total rather than instantaneous H_total·H_point·H_turb. Consequently, severe fading can push legitimate ones below threshold.

# 11. Receiver: Photodetection, Noise, Saturation and Decision

## 11.1 Photodetection

I_photo(t)=R·P_rx(t)

rxCurrent = R * rxOpticalSignal;

## 11.2 Receiver noise

n(t)~N(0,σ_noise²)

I_noisy=I_photo+n

Noise is generated outside receiverModel so experiments can reuse the exact same realization.

## 11.3 Hard saturation

I_out=min(I_noisy,I_sat)

if enableSaturation
    I_sat = 0.25;
    rxCurrent = min(rxCurrent,I_sat);
end

This hard-clipping model is central to the blinding studies.

## 11.4 Mid-bit sampling

sampleIndices = samplesPerBit/2 : samplesPerBit : length(rxCurrent);

## 11.5 Threshold and decision

I_th = R·P_t·H_total/2

b̂_k=1 if I_sample,k>I_th; otherwise 0

## 11.6 BER and error types

BER = errors/Nbits

Errors_10=count(b=1,b̂=0),  Errors_01=count(b=0,b̂=1)

The split between 1→0 and 0→1 errors becomes the main diagnostic tool for interpreting attack mechanisms.

# 12. The runBaseline Simulation Engine

runBaseline.m is intentionally the legitimate-system engine: no jammer is injected inside it.

| Step | Action |
| --- | --- |
| 1 | Set random seed. |
| 2 | Generate bits and OOK waveform. |
| 3 | Compute geometry. |
| 4 | Compute atmospheric attenuation. |
| 5 | Form H_total. |
| 6 | Generate and expand pointing gains. |
| 7 | Generate and expand turbulence gains. |
| 8 | Apply multiplicative legitimate channel. |
| 9 | Generate receiver noise. |
| 10 | Run receiver. |
| 11 | Store diagnostics, BER, error types and channel information. |

## Important returned fields

bits / detectedBits

txSignal / rxOpticalSignal

rxCurrent / rxSamples

receiverNoise

mean and standard deviation of samples for transmitted 0 and 1

BER, threshold, bitErrors, errors10, errors01

H_geo, H_atm, H_total, H_point_bits, H_turb_bits

wRx, w0, zR, alphaGG, betaGG, sigmaR2

Storing results.receiverNoise fixed an important development issue and enabled fair attacked-versus-clean comparisons.

# 13. Testing and Baseline Validation

## Test 01 — Perfect / Ideal Channel

params.sigmaPoint = 0;
params.Cn2 = 0;
params.sigmaNoise = 0;

This verifies that the modulation, channel plumbing, sampling and decision logic can operate correctly when impairments are removed.

## Test 02 — Receiver Noise Validation

The source uses a perfect channel and studies receiver-noise effects, with Nbits=100000 for more reliable BER statistics.

## Validation philosophy

| Question | Validation mechanism |
| --- | --- |
| Does the system work ideally? | Perfect-channel test. |
| Does noise hurt performance? | Noise validation. |
| Does distance affect propagation? | Distance sweep. |
| Does stronger turbulence degrade the link? | Cn² sweep. |
| Does misalignment degrade the link? | Pointing sweep. |
| Do gains actually vary? | Channel realization. |

# 14. Baseline Experiments: EXP_BASE_01 to EXP_BASE_04

## EXP_BASE_01 — Distance Sweep

The source sweeps distance from 1 km to 30 km. The intended physical trend is reduced received margin as propagation losses and beam spreading become more severe.

L↑ → received signal margin generally ↓ → BER eventually ↑

## EXP_BASE_02 — Turbulence Sweep

Distance and pointing are fixed while Cn² is varied to isolate atmospheric scintillation.

## EXP_BASE_03 — Pointing Sweep

Pointing-error standard deviation is varied to isolate misalignment-induced coupling loss.

## EXP_BASE_04 — Channel Realization

This experiment emphasizes that the channel contains random per-bit gains rather than a single constant number.

Together, these experiments establish the legitimate physical baseline before adversarial optical injection is introduced.

# 15. Threat Model: Optical Jamming

P_total(t)=P_legitimate(t)+P_jammer(t)

The attacker injects optical power at the receiver input abstraction. The receiver then converts the total optical power into electrical current.

| Attack family | Model |
| --- | --- |
| Constant jammer | P_jammer(t)=P_J continuously. |
| Random jammer | J_k~Bernoulli(p); P_jammer,k=P_ON·J_k. |

## Attack mechanisms

Positive optical bias can shift samples relative to threshold.

Strong bias can cause zeros to become false ones.

Sufficient optical power can trigger hard receiver saturation and create a blinding regime.

# 16. Constant Jammer and EXP 05–06

## jammerModel.m

if ~enableJammer
    jammerSignal=zeros(1,Nsamples);
else
    jammerSignal=jammerPower*ones(1,Nsamples);
end

P_jammer(t)=P_J

## EXP 05 — Controlled constant-jammer comparison

The experiment explicitly aims to preserve transmitted bits, pointing realization, turbulence realization and receiver noise so that the jammer is the intended changing factor.

## EXP 06 — Jammer power sweep

The sweep records BER and both error types. Development results exposed a non-monotonic curve: initially BER decreased as positive optical bias rescued faded ones; at larger power the attack created dominant 0→1 errors and BER approached 0.5.

Weak legitimate '1' below threshold → 1→0
Weak legitimate '1' + small jammer → may cross threshold correctly
Logical '0' + sufficiently strong jammer → crosses threshold → 0→1

The key lesson is that jammer power must be interpreted by mechanism, not by the simplistic rule 'more jammer always means more BER'.

# 17. Random Jammer and EXP 07–08

## randomJammerModel.m

J_k~Bernoulli(p)

P_jammer,k=P_ON·J_k

jammerBits = rand(1,Nbits) < activityProbability;
jammerPowerBits = jammerPower .* jammerBits;
jammerSignal = repelem(jammerPowerBits,samplesPerBit);

The jammer chooses an ON/OFF state once per bit and remains in that state across the bit's 20 samples.

## EXP 07 — Validation

A recorded 50-bit validation used requested p=0.30 and observed actual activity 0.38. This finite-sample deviation is expected.

Actual activity = number of ON bits / Nbits

## EXP 08 — Activity sweep

Activity probability is swept from 0 to 1 at fixed ON-power. The experiment demonstrates that activity is an independent attack control parameter and that the resulting BER depends on the receiver operating regime, threshold and saturation configuration.

# 18. EXP 09 — Constant Jammer Blinding Attack

EXP 09 uses Nbits=10000, enables saturation and sweeps jammer power into the overload region.

| Pj (W) | BER | 1→0 | 0→1 | Saturation |
| --- | --- | --- | --- | --- |
| 0.000 | 0.1177 | 1177 | 0 | 0.107 |
| 0.010 | 0.1006 | 1006 | 0 | 0.115 |
| 0.030 | 0.0643 | 643 | 0 | 0.133 |
| 0.050 | 0.0360 | 360 | 0 | 0.155 |
| 0.070 | 0.0151 | 151 | 0 | 0.177 |
| 0.100 | 0.0018 | 12 | 6 | 0.215 |
| 0.120 | 0.3026 | 0 | 3026 | 0.245 |
| 0.150 | 0.5076 | 0 | 5076 | 0.295 |
| 0.200 | 0.5076 | 0 | 5076 | 0.386 |
| 0.250 | 0.5076 | 0 | 5076 | 0.464 |
| 0.300 | 0.5076 | 0 | 5076 | 0.504 |
| 0.400 | 0.5076 | 0 | 5076 | 1.000 |
| 0.500 | 0.5076 | 0 | 5076 | 1.000 |
| 0.750 | 0.5076 | 0 | 5076 | 1.000 |
| 1.000 | 0.5076 | 0 | 5076 | 1.000 |

EXP 09: recorded BER versus constant jammer power.

EXP 09: recorded error-mechanism transition.

EXP 09: recorded receiver saturation fraction.

## Regime interpretation

| Region | Observed behavior | Mechanism |
| --- | --- | --- |
| Baseline/weak | BER begins near 0.118 | Mostly fading-driven 1→0 errors. |
| Helpful bias | BER falls to 0.0018 near 0.10 W | Bias rescues faded ones. |
| Transition | BER jumps at 0.12 W | 0→1 errors dominate. |
| Blinding plateau | BER ≈0.5076 | Receiver effectively outputs one for nearly all bits. |
| Full saturation | Sat=1 at high power | Hard clipping active for all samples. |

If b̂≈1 for all bits, BER≈fraction of transmitted zeros≈0.5

The recorded 0.5076 plateau corresponds to 5076 zero bits in the 10000-bit realization.

# 19. EXP 10 — Random Jammer Blinding Attack

EXP 10 fixes peak jammer power at 0.4 W and sweeps activity probability. The attacked signal is formed by adding the random jammer to the legitimate baseline signal, while the stored baseline receiver noise is reused.

Expected average jammer power = p·P_ON

| p | Actual | BER | 1→0 | 0→1 | Sat | Avg Pj |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 0.000 | 0.1193 | 1193 | 0 | 0.108 | 0.0000 |
| 0.1 | 0.103 | 0.1586 | 1066 | 520 | 0.201 | 0.0412 |
| 0.2 | 0.210 | 0.2043 | 959 | 1084 | 0.296 | 0.0840 |
| 0.3 | 0.308 | 0.2398 | 835 | 1563 | 0.384 | 0.1234 |
| 0.4 | 0.412 | 0.2805 | 717 | 2088 | 0.476 | 0.1648 |
| 0.5 | 0.515 | 0.3180 | 586 | 2594 | 0.568 | 0.2060 |
| 0.6 | 0.611 | 0.3575 | 473 | 3102 | 0.655 | 0.2444 |
| 0.7 | 0.710 | 0.3954 | 368 | 3586 | 0.744 | 0.2839 |
| 0.8 | 0.810 | 0.4306 | 241 | 4065 | 0.833 | 0.3239 |
| 0.9 | 0.904 | 0.4654 | 130 | 4524 | 0.915 | 0.3617 |
| 1.0 | 1.000 | 0.4990 | 0 | 4990 | 1.000 | 0.4000 |

EXP 10: BER increases progressively with random jammer activity.

EXP 10: actual activity and saturation fraction.

As activity rises, 1→0 errors fall while 0→1 errors rise strongly. At p=1 the random jammer is continuously ON and the BER reaches approximately 0.499 with full saturation.

# 20. EXP 11 — Fair Constant vs Random Jammer Comparison

This experiment controls the average optical power budget, making it the strongest comparison experiment before the ML phase.

## Fairness equation

P_avg=p·P_ON

p=P_avg/P_ON

The source fixes P_ON=0.4 W. The constant jammer uses P_avg continuously; the random jammer uses peak power 0.4 W with activity chosen from the equation above.

| Pavg | Random p | Actual random Pavg | BER const | BER random | Sat const | Sat random |
| --- | --- | --- | --- | --- | --- | --- |
| 0.00 | 0.000 | 0.0000 | 0.111 | 0.111 | 0.113 | 0.113 |
| 0.05 | 0.117 | 0.0468 | 0.033 | 0.161 | 0.165 | 0.220 |
| 0.10 | 0.239 | 0.0956 | 0.003 | 0.207 | 0.231 | 0.323 |
| 0.15 | 0.365 | 0.1460 | 0.494 | 0.247 | 0.310 | 0.432 |
| 0.20 | 0.495 | 0.1980 | 0.494 | 0.304 | 0.407 | 0.552 |
| 0.25 | 0.618 | 0.2472 | 0.494 | 0.349 | 0.480 | 0.659 |
| 0.30 | 0.740 | 0.2960 | 0.494 | 0.397 | 0.518 | 0.770 |
| 0.35 | 0.868 | 0.3472 | 0.494 | 0.454 | 1.000 | 0.883 |
| 0.40 | 1.000 | 0.4000 | 0.494 | 0.494 | 1.000 | 1.000 |

EXP 11: BER comparison under equal requested average power budget.

EXP 11: saturation comparison under equal requested average power budget.

## Interpretation

At Pavg=0.05 and 0.10 W, the constant jammer sits in the helpful-bias region and produces very low BER, while the random jammer remains damaging because every ON event has a 0.4 W instantaneous amplitude. At Pavg=0.15 W the constant jammer abruptly enters its false-one regime. The random jammer degrades more gradually as a larger fraction of bits experience the strong ON-state.

This is the project's strongest evidence that average power alone does not determine attack impact. Temporal distribution and instantaneous peak power matter.

# 21. Cross-Experiment Patterns and Physical Interpretation

## Pattern 1 — Non-monotonic BER

P_J↑ does not necessarily imply BER↑

Small positive bias can compensate for fading before becoming destructive.

## Pattern 2 — Error asymmetry explains the curve

Weak/no jammer: mostly 1→0
Helpful bias: 1→0 decreases
Transition: 0→1 begins
Strong/blinding: almost entirely 0→1

## Pattern 3 — Saturation and BER are different observables

Saturation can increase before BER reaches its maximum because BER depends on threshold crossings and symbol decisions.

## Pattern 4 — Temporal attack structure matters

Constant jamming shifts every bit; random jamming alternates between normal and high-power attack states.

## Pattern 5 — Blinding plateau near 0.5

An almost-always-one receiver makes every transmitted zero incorrect.

## Pattern 6 — Peak power matters

EXP 11 shows that a random attack can be damaging despite a moderate average power because its instantaneous ON-power is high.

# 22. Reproducibility, Fairness and Debugging Lessons

## Random seed control

Comparable simulations reset or fix RNG state so legitimate bits and channel realizations are reproducible.

## Same legitimate realization

A jammer comparison should not simultaneously change turbulence, pointing or bits.

## Same noise realization

runBaseline stores receiverNoise and attack experiments reuse it.

## receiverNoise bug

The MATLAB error 'Unrecognized field name receiverNoise' was solved by storing results.receiverNoise=receiverNoise.

## Finite statistics

Short validation runs can deviate substantially from requested probabilities; larger Nbits and Monte Carlo averaging improve confidence.

# 23. Model Assumptions and Limitations

OOK uses 0 W for bit 0 and Pt for bit 1.

Pointing and turbulence are independent block-fading realizations per bit.

The turbulence process is not explicitly time-correlated across bits.

Atmospheric attenuation is deterministic and path-averaged.

The jammer is injected at the receiver optical-input abstraction; its own propagation path is not modeled separately.

Receiver noise is additive Gaussian current noise.

Saturation is hard clipping at I_sat=0.25 A.

The threshold uses deterministic H_total rather than instantaneous fading estimates.

The receiver samples once at mid-bit rather than integrating over the full bit.

Single-run BER values retain finite-random-realization dependence.

These are appropriate simplifications for isolating causal mechanisms. A future version can increase physical realism after the ML objective is fixed.

# 24. Research Conclusions Through EXP 11

The project successfully builds a modular HAPS FSO communication chain from bits to BER.

Baseline fading primarily creates 1→0 errors under the fixed-threshold operating regime.

A small constant optical bias can temporarily improve BER by rescuing faded ones.

Beyond a transition point, optical injection creates dominant 0→1 errors.

Strong optical injection plus receiver saturation produces a blinding regime with BER near 0.5.

Random intermittent jamming is naturally controlled through a Bernoulli activity probability.

Random and constant jammers produce different error and saturation signatures.

Equal average jammer power does not imply equal damage because temporal structure and peak power matter.

The signatures discovered through EXP 11 provide a principled basis for ML attack detection and mitigation.

# 25. Bridge to Machine Learning

The next phase should build ML on the physical signatures already discovered, rather than choosing a model first.

## Possible tasks

| Task | Output |
| --- | --- |
| Attack detection | Normal vs attacked |
| Attack classification | No attack / constant / random / blinding |
| Severity classification | Weak / transition / severe / blinded |
| Mitigation control | Adaptive receiver response |

## Candidate features already available

| Feature | Meaning |
| --- | --- |
| meanBit0 / meanBit1 | Class-conditional received current levels. |
| stdBit0 / stdBit1 | Class-conditional variability. |
| BER | Aggregate degradation. |
| errors10 / errors01 | Mechanism asymmetry. |
| Saturation fraction | Receiver overload indicator. |
| Mean/max current | Operating level. |
| Threshold | Nominal decision boundary. |
| Channel statistics | Legitimate propagation context. |

## Recommended next architecture

Simulation scenarios
      ↓
Labelled dataset generation
      ↓
Feature extraction
      ↓
Train / validation / test split
      ↓
Baseline ML models
      ↓
Attack detection + classification
      ↓
Feature importance / interpretability
      ↓
Mitigation decision
      ↓
Adaptive receiver strategy

Do not train on BER alone. BER hides the difference between fading, helpful bias, transition and blinding. The ML dataset should preserve receiver-level statistics and error asymmetry.

# Appendix A. File-by-File Code Map

| File | Role |
| --- | --- |
| defaultParameters.m | Central nominal parameter factory. |
| main_baseline.m | Normal baseline entry point. |
| runBaseline.m | Complete legitimate simulation engine and results container. |
| generateOOK.m | Bits, OOK waveform and time axis. |
| geometryModel.m | Gaussian beam geometry and aperture coupling. |
| atmosphericAttenuation.m | dB/km to linear optical gain. |
| pointingErrorModel.m | Per-bit pointing displacement and coupling loss. |
| gammaGammaTurbulence.m | Rytov/Gamma-Gamma fading. |
| receiverModel.m | Photodetection, noise, saturation, sampling, threshold, BER. |
| jammerModel.m | Constant optical jammer. |
| randomJammerModel.m | Bernoulli intermittent jammer. |
| test01_perfectChannel.m | Ideal-channel validation. |
| test02_receiverNoise.m | Noise sensitivity validation. |
| EXP_BASE_01_distanceSweep.m | Distance study. |
| EXP_BASE_02_turbulenceSweep.m | Turbulence study. |
| EXP_BASE_03_pointingSweep.m | Pointing study. |
| EXP_BASE_04_channelRealization.m | Channel realization study. |
| EXP_05_constantJammer.m | Controlled constant-jammer comparison. |
| EXP_06_jammerPowerSweep.m | Constant jammer sweep. |
| EXP_07_randomJammerValidation.m | Random activity validation. |
| EXP_08_randomJammerSweep.m | Random activity sweep. |
| EXP_09_constantJammerBlindingAttack.m | Constant blinding sweep. |
| EXP_10_randomJammerBlindingAttack.m | Random blinding sweep. |
| EXP_11_fairJammerComparison.m | Equal-average-power comparison. |

# Appendix B. Key Equations

| Quantity | Equation |
| --- | --- |
| Bit duration | T_b=1/R_b |
| Sampling frequency | F_s=R_b·samplesPerBit |
| OOK | P_tx,k=P_t b_k |
| Beam waist | w_0=λ/(πθ) |
| Rayleigh range | z_R=πw_0²/λ |
| Beam radius | w_Rx=w_0√(1+(L/z_R)²) |
| Geometric gain | H_geo=1−exp(−2R_Rx²/w_Rx²) |
| Atmospheric gain | H_atm=10^(−αL_km/10) |
| Deterministic gain | H_total=H_geoH_atm |
| Pointing gain | H_point=exp(−2r_e²/w_Rx²) |
| Rytov variance | σ_R²=1.23Cn²k^(7/6)L^(11/6) |
| Turbulence | H_turb=XY |
| Legitimate optical signal | P_legit=P_txH_totalH_pointH_turb |
| Attacked signal | P_total=P_legit+P_jammer |
| Photocurrent | I=RP_total+n |
| Saturation | I_out=min(I,I_sat) |
| Threshold | I_th=RP_tH_total/2 |
| BER | BER=errors/Nbits |
| Random average power | P_avg=pP_ON |
| Fair activity | p=P_avg/P_ON |

# Appendix C. Key Experimental Results

## EXP 09

Pj=0.000 W → BER 0.1177, mainly 1→0
Pj=0.100 W → BER 0.0018, helpful-bias region
Pj=0.120 W → BER 0.3026, strong 0→1 transition
Pj≥0.150 W → BER≈0.5076, false-one/blinding plateau
Pj≥0.400 W → saturation fraction=1.0 in recorded run

## EXP 10

Activity 0.0 → BER 0.1193, Sat 0.108
Activity 0.5 → BER 0.3180, Sat 0.568
Activity 0.8 → BER 0.4306, Sat 0.833
Activity 1.0 → BER 0.4990, Sat 1.000

## EXP 11

Pavg 0.05 W: Constant BER 0.033, Random BER 0.161
Pavg 0.10 W: Constant BER 0.003, Random BER 0.207
Pavg 0.15 W: Constant BER 0.494, Random BER 0.247
Pavg 0.30 W: Constant BER 0.494, Random BER 0.397
Pavg 0.40 W: Constant BER 0.494, Random BER 0.494

# Final project narrative

The project has progressed from first-principles optical communication to an experimentally characterized physical-layer security problem. The baseline explains how legitimate information is transmitted and degraded. The jammer experiments reveal multiple attack regimes rather than a single monotonic failure mechanism. The blinding experiments connect excessive optical power to receiver saturation and information collapse. The fair comparison establishes that temporal attack structure and instantaneous peak power matter even under equal average power budgets. These signatures provide the correct foundation for the next ML phase.