HAPS → Ground FSO Communication System
Consolidated Baseline Model, Assumptions, Equations, Program Flow & Architecture

Project focus: Optical blinding/jamming and mitigation
Status: Baseline channel frozen before jammer implementation
Simulation platform: MATLAB

# 0. Purpose of This Document

This document consolidates the legitimate HAPS-to-ground FSO baseline built and validated before introducing the optical blinding jammer. It traces the system from random bits to OOK waveform, through geometric propagation, atmospheric attenuation, Gamma-Gamma turbulence, pointing error, photodetection, receiver noise, saturation, threshold detection and BER.

The baseline is intentionally realistic enough for the attack/mitigation study while remaining a tractable engineering simulation. It is not a complete hardware-accurate or atmospheric digital twin.

# 1. High-Level Architecture

HAPS optical transmitter
        ↓
Random bits
        ↓
OOK modulation
        ↓
Sampled optical waveform
        ↓
Gaussian beam / geometric coupling
        ↓
Atmospheric attenuation
        ↓
Gamma-Gamma turbulence
        ↓
Pointing-error loss
        ↓
Received optical power
        ↓
Photodetector (R × P)
        ↓
Additive receiver noise
        ↓
Photodetector saturation
        ↓
Sample once per bit
        ↓
Fixed threshold
        ↓
Detected bits
        ↓
BER

Future attack architecture:

HAPS legitimate optical signal ───────┐
                                        ↓
                              Optical power summation
                                        ↑
Jammer optical signal ──────────────────┘
                                        ↓
                              Photodetector + noise
                                        ↓
                                    Saturation
                                        ↓
                                  Decision / BER
                                        ↓
                               Jammer detection / ML
                                        ↓
                                   Mitigation

# 2. Time and Bit Model

Bits describe information. samplesPerBit describes numerical time resolution for MATLAB. The current bit rate is Rb = 1 Mbps, so Tb = 1/Rb = 1 microsecond. With 20 samples/bit, Fs = Rb × samplesPerBit = 20 MHz.

| Parameter | Value | Meaning |
| --- | --- | --- |
| Nbits (debug) | 20 | Small waveform run |
| NbitsSweep | 10000 | BER sweep |
| Rb | 1e6 bit/s | Bit rate |
| samplesPerBit | 20 | Numerical resolution |
| Fs | 20 MHz | Sampling frequency |
| Tb | 1 µs | Bit duration |

# 3. Why 20 Samples per Bit?

20 samples/bit is not a physical HAPS parameter. It gives MATLAB enough time-domain points to represent a rectangular OOK bit and to apply/visualize channel effects. One bit is represented by 20 samples. It must not be confused with channel variation.

One bit:
sample 1 ... sample 20
       └── same Hturb[k] and Hpoint[k] ──┘

For N bits, total waveform samples = N × 20. In the current block-fading approximation, turbulence and pointing each have one realization per bit, repeated across the 20 samples.

# 4. Transmitter: Bits → OOK

bits = randi([0 1],1,Nbits)
Ptx[k] = Pt × bits[k]
txSignal = repelem(Ptx, samplesPerBit)
t[n] = n/Fs

OOK therefore transmits 0 W for bit 0 and Pt = 1 W for bit 1. repelem expands each symbol into 20 samples.

# 5. Gaussian Beam Geometry

λ = 1550 nm

θ = 0.01 mrad half-angle

Receiver diameter = 0.2 m

Receiver radius = 0.1 m

HAPS-ground distance L = 20 km

w0  = λ/(πθ)
zR  = πw0²/λ
wRx = w0√(1+(L/zR)²)
Hgeo = 1 − exp(−2RRx²/wRx²)

At 20 km the current run gives Hgeo = 0.375821, wRx = 0.2060 m, w0 = 0.0493 m and zR = 4.9338 km.

# 6. Atmospheric Attenuation

α = 0.1 dB/km
Lkm = L/1000
Hatm = 10^(−αLkm/10)

At 20 km, Hatm = 0.630957. This is a path-averaged deterministic attenuation model, not a height-dependent atmospheric profile.

# 7. Gamma-Gamma Atmospheric Turbulence

Turbulence is represented by a random multiplicative gain Hturb. The current gammaGammaTurbulence.m function maps Cn² to Rytov variance and Gamma-Gamma parameters, then generates normalized fading.

Cn² → Rytov variance σR² → αGG, βGG → Hturb

At Cn² = 1×10^-15 m^-2/3, the current implementation gives σR² = 4.833736, αGG = 4.5400 and βGG = 1.2461. Over many samples the normalized gain has mean near unity. Stronger Cn² produces stronger fluctuations.

Assumption: one turbulence realization per bit, repeated over that bit. This is a block-fading approximation, not a fully time-correlated atmospheric process.

# 8. Pointing Error

xe ~ N(0,σp²)
ye ~ N(0,σp²)
re = √(xe²+ye²)
Hpoint = exp(−2re²/wRx²)

Current σp = 0.02 m. A representative run produced mean Hpoint ≈ 0.964, with 0.78–1.00 in one 20-bit realization. One pointing realization is generated per bit and repeated across its 20 waveform samples.

# 9. Combined Legitimate Optical Channel

Plegit[k] = Ptx[k] × Hgeo × Hatm × Hturb[k] × Hpoint[k]

For bit 0, Ptx = 0, so the ideal optical contribution is zero. For bit 1, the instantaneous received power is Pt × Hgeo × Hatm × Hturb[k] × Hpoint[k]. The deterministic component is Hdet = Hgeo × Hatm = 0.237127.

# 10. Receiver

Iphoto[k] = R × Plegit[k]
Inoise[k] = σn × randn()
Inoisy[k] = Iphoto[k] + Inoise[k]
Iout[k] = min(Inoisy[k], Isat)

Responsivity R = 0.8 A/W

Noise standard deviation σn = 0.005 A

Saturation current Isat = 0.25 A

The hard saturation model is especially important for the future optical blinding attack because the jammer will add optical power before the photodetector.

# 11. Threshold and BER

Ith = R × Pt × Hdet / 2
I1  = R × Pt × Hdet
detectedBit[k] = (Iout[k] > Ith)
BER = errors/Nbits

Current values: I1 = 0.189702 A and Ith = 0.094851 A. The threshold is fixed at half the nominal 1-current. It does not adapt to instantaneous turbulence or pointing loss.

Without noise and pointing, the threshold condition for a transmitted 1 reduces to Hturb > 0.5. With noise and pointing, the exact decision depends on all three effects.

# 12. Validated Behavior

Gaussian geometric coupling decreases with increasing propagation distance.

Atmospheric attenuation reduces received power with distance.

Gamma-Gamma turbulence produces normalized random fading; stronger Cn² gives stronger fluctuations.

Applying turbulence produces received-power fluctuations and detection errors.

Pointing error produces a gain between 0 and 1 and reduces legitimate received power.

Receiver noise is present without destroying the nominal baseline under the selected parameter.

Receiver saturation is below the future jammer operating region while the nominal legitimate 1 remains below saturation.

The combined turbulence + pointing BER sweep increases as turbulence strength increases.

Representative combined sweep: Cn² = 1e-17 → BER ≈ 0.0021; 3e-17 → 0.0353; 1e-16 → 0.1182; 3e-16 → 0.1733; 1e-15 → 0.2147. Exact values vary because each run uses random realizations.

# 13. MATLAB Module Architecture

| File | Responsibility | Main interface |
| --- | --- | --- |
| main.m | Experiment orchestration, parameters, plots, sweeps | Calls all modules |
| generateOOK.m | Bits and sampled OOK waveform | Nbits, Pt, Rb, samples/bit → bits, txSignal, t |
| geometryModel.m | Gaussian beam and aperture coupling | λ, θ, aperture, L → Hgeo, wRx, w0, zR |
| atmosphericAttenuation.m | Deterministic dB/km loss | α, L → Hatm |
| gammaGammaTurbulence.m | Turbulence statistics and random gain | λ, L, Cn², N → Hturb, αGG, βGG, σR² |
| pointingErrorModel.m | Per-bit pointing loss | wRx, σp, Nbits → Hpoint |
| receiverModel.m | Photodetection, noise, saturation, sampling, threshold, BER | rx optical signal + receiver parameters → detected bits, BER |

# 14. Program Flow

1. main.m clears the workspace and defines system parameters.

2. generateOOK.m creates the random information bits and sampled OOK waveform.

3. geometryModel.m computes Gaussian beam geometry and receiver-aperture coupling.

4. atmosphericAttenuation.m computes deterministic path loss.

5. gammaGammaTurbulence.m generates one turbulence gain per bit.

6. pointingErrorModel.m generates one pointing-loss gain per bit.

7. repelem() expands each per-bit gain over the 20 waveform samples.

8. The gains multiply the OOK optical waveform to form the legitimate received optical signal.

9. receiverModel.m converts optical power to current, adds noise, clips at saturation, samples once per bit, applies the threshold and computes BER.

10. The turbulence sweep repeats the experiment for several Cn² values and stores BER.

# 15. Formula Chaining: From Bits to BER

bits[k] ∈ {0,1}
        ↓
Ptx[k] = Pt·bits[k]
        ↓
w0 = λ/(πθ)
zR = πw0²/λ
wRx = w0√(1+(L/zR)²)
Hgeo = 1−exp(−2RRx²/wRx²)
        ↓
Hatm = 10^(−αLkm/10)
        ↓
Hturb[k]  ← Gamma-Gamma(Cn²,λ,L)
Hpoint[k] = exp(−2re[k]²/wRx²)
        ↓
Plegit[k] = Ptx[k]HgeoHatmHturb[k]Hpoint[k]
        ↓
Iphoto[k] = R·Plegit[k]
        ↓
Inoisy[k] = Iphoto[k]+n[k]
        ↓
Iout[k] = min(Inoisy[k],Isat)
        ↓
detected[k] = 1 if Iout[k] > Ith
        ↓
BER = errors/Nbits

For the future jammer, the only major addition is optical power before the detector:

Ptotal[k] = Plegit[k] + Pjam[k]
        ↓
Iphoto[k] = R·Ptotal[k]
        ↓
noise → saturation → threshold → BER/outage

# 16. Assumptions and Scope

The studied architecture is HAPS → ground receiver.

OOK intensity modulation is used.

λ = 1550 nm and Pt = 1 W.

Receiver aperture diameter = 0.2 m.

Gaussian beam spreading and circular-aperture coupling are modeled explicitly.

Atmospheric attenuation uses a constant path-averaged α = 0.1 dB/km.

Turbulence uses the existing Gamma-Gamma implementation.

Turbulence is block fading: one realization per bit.

Pointing error uses Gaussian x/y displacement and a Gaussian beam-coupling loss, one realization per bit.

Receiver noise is additive zero-mean Gaussian current noise.

Receiver saturation is a hard current ceiling.

The threshold is fixed at half the nominal received 1-current.

The current model does not use a height-dependent Cn²(h) profile.

The current model does not use detailed temporal turbulence correlation.

The current model does not use detailed detector-specific shot/thermal-noise physics.

Fog, rain, polarization, laser linewidth and secondary hardware nonlinearities are outside the present scope.

These simplifications are intentional so the main jamming/mitigation study remains interpretable and computationally manageable.

# 17. What We Are NOT Claiming

We are not claiming that the current model predicts exact real-world HAPS link BER for every atmospheric condition. We are claiming that it provides a representative, parameterized HAPS-to-ground FSO channel with the major impairments that interact with the planned optical blinding attack.

The model is therefore suitable for controlled comparative studies: baseline versus jammer, and jammer versus mitigation, under the same legitimate-channel conditions.

The attack study will quantify BER, outage probability, saturation behavior and how attack parameters alter receiver performance. ML will be introduced after the attack's measurable signatures are established, so the ML system has a clear engineering purpose.

# 18. Key Mental Model

There are three distinct levels. Information level: bits → OOK. Channel-physics level: geometry × attenuation × turbulence × pointing. Receiver level: optical power → current → noise → saturation → threshold → BER.

The jammer is a fourth experimental input: an additional optical-power source. This separation allows the exact same legitimate HAPS FSO baseline to be compared before attack, during attack, and after mitigation.

# 19. One Critical Interpretation Rule

Hdet = 0.237127 is the deterministic gain from geometry and attenuation for the fixed 20-km path. It is not the instantaneous gain of every bit. The actual per-bit gain is Hdet × Hturb[k] × Hpoint[k]. With 10,000 bits and 20 samples/bit, there are 10,000 channel realizations represented across 200,000 waveform samples.

END OF FROZEN BASELINE — NEXT: OPTICAL BLINDING JAMMER